Universal Asynchronous Receiver Transmitter

# Introduction

UART, or Universal Asynchronous Receiver/Transmitter, is a full duplex hardware communication protocol used for serial data transmission between two devices. It's a simple, two-wire system (transmit and receive) that's widely used for device-to-device communication, particularly in embedded systems. UARTs handle the conversion between parallel data (used by the controlling device) and serial data (transmitted over the communication lines).

Contents

Introduction	2

UART Transmitter	4

# UART Transmitter

Block Diagram

| Port | Direction | Width (bits) | Description |
| --- | --- | --- | --- |
| P_DATA | INPUT | 8 | Input data byte |
| DATA_VALID | INPUT | 1 | High, when data is available on the data bus (P_DATA) |
| PAR_EN | INPUT | 1 | High to enable parity calculation |
| PAR_TYPE | INPUT | 1 | Parity Type 0: Even Parity bit 1: Odd Parity bit |
| CLK | INPUT | 1 | Input clk signal |
| RST_N | INPUT | 1 | Active low Asynchronous reset signal |
| TX_OUT | OUTPUT | 1 | Serial output data line |
| BUSY | OUTPUT | 1 | High signal during transmission, otherwise low |

Verilog Modules

- Serializer

module serializer(

input clk,

input rst_n,

input ser_en,

input [7:0] P_DATA,

output reg ser_done,

output reg ser_data

);

integer counter;

reg [7:0] shift_reg;

always @(posedge clk, negedge rst_n) begin

if (!rst_n) begin

shift_reg <= P_DATA;

counter   <= 0;

ser_data  <= 1'b1; // IDLE line

ser_done  <= 1'b0;

end

else if (ser_en) begin

ser_data  <= shift_reg[0];

shift_reg <= shift_reg >> 1;

counter   <= counter + 1;

ser_done <= (counter >= 7);

end

else begin

shift_reg <= P_DATA;

ser_data  <= 1'b1; // IDLE line

end

end

- FSM

module fsm (

input Data_Valid,

input PAR_EN,

input ser_done,

input clk, rst_n,

output reg ser_en,

output reg busy,

output reg [1:0] mux_sel

);

localparam [2:0] IDLE   = 3'b000,

START  = 3'b001,

PARITY = 3'b011,

STOP   = 3'b010;

reg [2:0] ps, ns;

// state memory

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

ps <= IDLE;

end

else

ps <= ns;

end

// transion and output logic

always @(*) begin

ns = ps;

ser_en = 0;

busy = 0;

mux_sel = 2'b10;

case (ps)

IDLE: begin

if (Data_Valid) begin

ns = START;

mux_sel = 2'b00; // start bit

busy = 1;

ser_en = 1;

end

else

ns = IDLE;

end

START: begin

busy = 1;

mux_sel = 2'b10; // Send data

if (ser_done) begin

ns = PAR_EN? PARITY : STOP;

end else begin

ser_en = 1;

ns = START;

end

end

PARITY: begin

busy = 1;

mux_sel = 2'b11; // parity bit

if (ser_done)

ns = STOP;

else

ser_en = 1;

end

STOP: begin

busy = 0;

mux_sel = 2'b01; //stop bit

if (ser_done)

ns = IDLE;

else

ser_en = 1;

end

default: ns = IDLE;

endcase

end

endmodule

- Parity Calc

module parity_calc(

input PAR_EN,

input [7:0] P_DATA,

input PAR_TYPE,

output reg par_bit

);

always @(*) begin

if (!PAR_EN)

par_bit = 0;

else

par_bit = (PAR_TYPE)? ~(^P_DATA) : (^P_DATA); // xor reduction

end

endmodule

- MUX

module mux_4to1(

input [3:0] I,

input [1:0] mux_sel,

output Y

);

assign Y = I[mux_sel];

endmodule

- UART_tx (Top)

module uart_tx(

input [7:0] P_DATA,

input DATA_VALID,

input PAR_EN, PAR_TYP,

input clk, rst_n,

output TX_OUT, Busy

);

wire ser_en, ser_done, ser_data, par_bit;

wire [1:0] mux_sel;

serializer ser (clk,rst_n,ser_en,P_DATA,ser_done,ser_data);

parity_calc par (PAR_EN, P_DATA, PAR_TYP, par_bit);

fsm FSM (DATA_VALID,PAR_EN,ser_done,clk,rst_n,ser_en, Busy, mux_sel);

mux_4to1 mux ({par_bit, ser_data, 1'b1, 1'b0},mux_sel,TX_OUT);

endmodule

Results and Waveform

- Test case 1: Send 8'b10101010, no parity

initial begin

rst_n = 0; DATA_VALID = 0; PAR_EN = 0; PAR_TYP = 0; P_DATA = 8'b0;

#10

P_DATA = 8'b10101010;

#10

rst_n = 1;

// Frame 1: Send 8'b10101010, no parity

#10

DATA_VALID = 1;

PAR_EN = 0;

#10

DATA_VALID = 0; // one clk cycle high

#150

Stop

IDLE

IDLE

0

0

0

1

1

1

1

0

Start

- Test Case 2: Send 8'b01001101 with odd parity

// Frame 2: Send 8'b01001101 with odd parity

rst_n = 0; DATA_VALID = 0; PAR_EN = 0; PAR_TYP = 0;

#10;

P_DATA = 8'b01001101;

#10

rst_n = 1;

#10

DATA_VALID = 1;

PAR_EN = 1;

PAR_TYP = 1;

#5 DATA_VALID = 0;

#100;

$stop;

end